PAN PACIFIC MASTERS GAMES – PPMG26

BROADBEACH CULTURAL CENTRE

LIGHTING REFERENCE MENU

Broadbeach Cultural Centre has a basic in-house lighting rig — a full stage colour wash and a set of gobos. There is no Front Light plot Motion, Foot Light, Blinders or Follow Spot at this venue. Only the cues listed below will be available.

## Lighting Limits

- Maximum of 8 cues per item

- Final cue (cue 8) must be a blackout

- Preset is by default a blackout

- For each cue, list only what needs to be ON. Anything not listed in that cue will automatically turn OFF from the previous cue.

## For Each Cue, Select:

- Up to ONE Wash Colour (C)

- Up to ONE Gobo (S)

Only the percentages listed in this menu are available — there is no in-between. If a chosen look reads too dark on the day, the venue's lighting technician may be asked to lift the facelight, but only using white light.

Timing

- Leave a minimum of TEN (10) seconds between cues.

- All cue changes take 1.5 seconds to complete once called.

Venue notes: stage is 10.7m deep x 10m wide. There is no back curtain to enter or exit the stage. Spotlights are not available.

## Wash Colour — select up to ONE per cue

Colours available on Broadbeach's in-house fixture (combined from its two colour wheels). Photos are from the ACC26 Lighting Reference Menu, used where the colour is the same or closest match — Broadbeach's own fixture may render slightly differently.

| Code | Photo | Colour |
| --- | --- | --- |
| C1 |  | Red |
| C2 |  | Orange |
| C3 |  | Yellow |
| C4 | — | Dark Yellow (Amber) — no ACC26 equivalent photo |
| C5 |  | Green |
| C6 |  | Light Blue (closest match shown: Aqua) |
| C7 |  | Dark Blue |
| C8 |  | Magenta / Pink |
| C9 |  | UV / Purple |
| C10 |  | Warm White (3200K) |
| C11 |  | Cool White (5600K) |

## Gobo Spot — select up to ONE per cue

Broadbeach's in-house gobo library — Static (fixed) and Rotating (indexing) wheels.

| Code | Image | Description | Type |
| --- | --- | --- | --- |
| S1 |  | Dots Breakup | Static |
| S2 |  | Bricks | Static |
| S3 |  | Triangle Rays | Static |
| S4 |  | Grouped Ovals | Static |
| S5 |  | Triangle Breakup | Static |
| S6 |  | Foliage Breakup | Static |
| S7 |  | Circle Dot | Static |
| S8 |  | Mosaic Breakup | Static |
| S9 |  | Tribal Breakup | Rotating |
| S10 |  | Shattered | Rotating |
| S11 |  | Stickmen | Rotating |
| S12 |  | Linear Ovals | Rotating |
| S13 |  | Laser Triangle | Rotating |
| S14 |  | Line | Rotating |
| S15 |  | Thatched Breakup | Rotating |